Subject: 10.7.3.4 Aerodrom Control Service
Question: 401 / 497 (ID: 4649)

Special VFR flights may be authorized to operate locally within a control zone when the ground visibility is not less than 1 500 metres, even when the aircraft is not equipped with a functioning radio receiver within class:

a) C, D and E airspaces
b) E airspace
c) D airspace
d) D and E airspaces

Subject: 10.7.3.4 Aerodrom Control Service
Question: 402 / 497 (ID: 4650)

In order to meet wake turbulence criteria, for arriving aircraft and using timed approaches, what minima shall be applied to aircraft landing behind a heavy or a medium aircraft?

a) medium aircraft other medium aircraft - 2 minutes
b) medium aircraft behind heavy aircraft - 3 minutes
c) medium aircraft behind heavy aircraft - 2 minutes
d) light aircraft behind medium aircraft 4 minutes

Subject: 10.7.3.4 Aerodrom Control Service
Question: 403 / 497 (ID: 4652)

A separation minimum shall be applied between a light or MEDIUM aircraft and a HEAVY aircraft and between a LIGHT aircraft and a MEDIUM aircraft when the heavier aircraft is making a low or missed approach and the lighter aircraft is landing on the same runway in the opposite direction or on a parallel opposite direction runway separated by:

a) Less than 730 m
b) 730 m
c) Less than 760 m
d) 760 m

Subject: 10.7.3.4 Aerodrom Control Service
Question: 404 / 497 (ID: 4653)

A separation minimum shall be applied between a light or MEDIUM aircraft and a HEAVY aircraft and between a LIGHT aircraft and a MEDIUM aircraft when the heavier aircraft is making a low or missed approach and the lighter aircraft is utilizing an opposite direction runway for take off, this minimum is:

a) 2 minutes
b) 1 minute
c) 3 minutes
d) 5 minutes

Subject: 10.7.3.4 Aerodrom Control Service
Question: 405 / 497 (ID: 4660)

In order to meet the wake turbulence criteria, what minimum separation should be applied when a medium aircraft is taking off behing a heavy aircraft and both are using the same runway?

a) 1 minute
b) 3 minutes
c) 2 minutes
d) 4 minutes

Subject: 10.7.3.5 Flight Information Service and Alerting Service
Question: 406 / 497 (ID: 1119)

Flight information service shall be provided to all aircraft which are likely to be affected by the information and which are:

a) Provided with air traffic control services, only
b) Provided with the air traffic control services and otherwise known to the relevant air traffic service units
c) Known to the relevant air traffic services units
d) Known to the relevant air traffic services units by a filed flight plan

Subject: 10.7.3.5 Flight Information Service and Alerting Service
Question: 407 / 497 (ID: 1120)

Alerting service shall be provided:

a) For all controlled flight, to any aircraft known or believed to be subject of unlawful interference, and in so far as practicable to all aircraft having filed a flight plan or otherwise known to the ATS
b) To any aircraft known or believed to be subject of unlawful interference, only
c) For all aircraft provided with air traffic control services, only
d) In so far as practicable to all aircraft having filed a flight plan or otherwise known by the ATS

Subject: 10.7.3.6 Use of radar in ATS
Question: 408 / 497 (ID: 1121)

Upon intercepting the assigned radial, the controller advises you that you are on the airway and to "resume own navigation". This phrase means that:

a) You are still in radar contact, but must make position reports
b) You are to contact the centre at the next reporting point
c) You are to assume responsability for your own navigation
d) Radar services are terminated and you will be responsable for position reports

Subject: 10.7.3.6 Use of radar in ATS
Question: 409 / 497 (ID: 1122)

The Air Traffic control Services: do not prevent collisions with terrarin.

a) Prevent collisions with terrain
b) Do not prevent collisions with terrain
c) Correct, except when an IFR flight is vectored by radar
d) Except when an aircraft is flying IFR in IMc

Subject: 10.7.3.6 Use of radar in ATS
Question: 410 / 497 (ID: 1123)

Which code shall be used on mode "A" to provide recognition of an emergency aircraft?

a) Code 7700
b) Code 7000
c) Code 7500
d) Code 7600

Subject: 10.7.3.6 Use of radar in ATS
Question: 411 / 497 (ID: 1124)

One of the functions ensured by a radar control unit for the provision of approach control service is:

a) To apply a reduced vertical separation of 500 feet between IFR flights and VFR flights
b) To provide instructions in order to reduce separations minima, if accepted by the pilots
c) To apply a horizontal separation less than 5 NM
d) To conduct surveillance radar approaches

Subject: 10.7.3.6 Use of radar in ATS
Question: 412 / 497 (ID: 1125)

The primary duty provided by a radar unit is:

a) To provide radar separation
b) To assist aircraft where navigation appears unsatisfactory
c) To assist aircraft on the location storms
d) To assist aircraft due to failure of airborne equipment

Subject: 10.7.3.6 Use of radar in ATS
Question: 413 / 497 (ID: 1126)

When radar identification of aircraft has been achieved, ATC unit shall:

a) Inform the aircraft only if radar identification has been achieved without availability of SSR
b) Inform the aircraft prior to issue any instructions or advice based on the use of radar
c) not advise the aircraft before issuing instructions
d) Inform the aircraft only if communication's load permits it

Subject: 10.7.3.6 Use of radar in ATS
Question: 414 / 497 (ID: 1127)

One of the functions ensured by a radar control unit for the provision of approach control service is:

a) To conduct precision radar approach (PAR)
b) To apply a reduced vertical separation of 500 feet between IFR and VFR
c) To apply a horizontal separation less than 5 NM
d) To provide instructions to reduce the separation minima

Subject: 10.7.3.6 Use of radar in ATS
Question: 415 / 497 (ID: 1128)

Except otherwise established by the appropriate ATS authority a Surveillance Radar Approach (SRA) shall be terminated at a distance from the touchdown of:

a) 5 NM
b) 2 NM
c) 3 NM
d) 4 NM

Subject: 10.7.3.6 Use of radar in ATS
Question: 416 / 497 (ID: 1129)

When "Secondary Radar" is used, an aircraft may be identified by one of the following procedures:

a) Observation of compliance with an instruction to operate transponder from "ON" to "STBY" and back to "ON"
b) To request pilot to set transponder on position "ON"
c) To request pilot to set transponder on position "OFF"
d) To request pilot to switch from "ON" to "STDBY"

Subject: 10.7.3.6 Use of radar in ATS
Question: 417 / 497 (ID: 1130)

Where a "Secondary Surveillance Radar" (SSR) is not available, radar identification may be achieved by one of the following procedures:

a) To instruct the pilot to execute one or more changes of 10°
b) To instruct the pilot to execute one or more changes of 45°
c) To instruct the pilot to execute one or more changes of 30° or more
d) To instruct the pilot to execute one or more changes of 20° or more

Subject: 10.7.3.6 Use of radar in ATS
Question: 418 / 497 (ID: 1131)

Which code shall be used on Mode "A" to provide recognition of an aircraft subjected to unlawful interference?

a) Code 7700
b) Code 7600
c) Code 2000
d) Code 7500

Subject: 10.7.3.6 Use of radar in ATS
Question: 419 / 497 (ID: 1132)

Which does ATC Term "Radar contact" signify?

a) Your aircraft has been identified on the radar display and radar flight instructions will be provided until radar identification is terminated
b) You will be given traffic advisories until advised that the service has been terminated or that radar contact has been lost
c) ATC is receiving your transponder and will furnish vectors and traffic advisories until you are advised that contact has been lost
d) Your aircraft has been identified and you will receive separation from all aircraft while in contact with this radar facility

Subject: 10.7.3.6 Use of radar in ATS
Question: 420 / 497 (ID: 1133)

What is meant when departure control instruct you to "resume own navigation" after you have been vectored to an airway?

a) You are still in radar contact, but must make position reports
b) You should maintain that airway by use of your navigation equipment
c) Radar Service is terminated
d) Advisories will no longer be issued by ATc

Subject: 10.7.3.6 Use of radar in ATS

Question: 421 / 497 (ID: 1768)

When vectoring an aircraft to intercept the localizer course, the final vector furnished shall be such as to enable the aircraft to intercept the localizer course at an angle not greater than:

a) 25 degrees
b) 30 degrees
c) 20 degrees
d) 15 degrees

Subject: 10.7.3.6 Use of radar in ATS
Question: 422 / 497 (ID: 1769)

The following minimum radar separation shall be provided between aircraft on the same localizer with additional longitudinal separation as required for wake turbulence:

a) 3 NM
b) 5 NM
c) 2.5 NM
d) 2 NM

Subject: 10.7.3.6 Use of radar in ATS
Question: 423 / 497 (ID: 1770)

The minimum radar separation to be provided to aircraft established on the localizer course shall be:

a) 2.0 NM between aircraft on the same localizer course
b) 3.0 NM between aircraft on the same localizer course
c) 5.0 NM between aircraft on the same localizer course
d) 3.0 NM between aircraft on adjacent localizer course

Subject: 10.7.3.6 Use of radar in ATS
Question: 424 / 497 (ID: 1771)

The tolerance value used to determine that mode C derived level information displayed to the controller is accurate shall be:

a) +/- 250 ft
b) +/- 300 ft
c) +/- 500 ft
d) +/- 200 ft

Subject: 10.7.3.6 Use of radar in ATS
Question: 425 / 497 (ID: 1772)

Unless otherwise prescribed by the appropriate ATS authority, the horizontal radar separation minimum shall be:

a) 3.0 NM
b) 3.5 NM
c) 5.0 NM
d) 10.0 NM

Subject: 10.7.3.6 Use of radar in ATS
Question: 426 / 497 (ID: 1773)

The criterion which shall be used to determine that a specific level is occupied by an aircraft shall be, (except that appropriate ATS authorities may specify a smaller criterion):

a) +/- 200 ft
b) +/- 250 ft
c) +/- 300 ft
d) +/- 150 ft

Subject: 10.7.3.6 Use of radar in ATS
Question: 427 / 497 (ID: 1839)

An aircraft in climb or descent is considered to have crossed a level when the SSR mode C derived level information indicates that it has passed this level in the required direction by:

a) +/- 300 ft
b) 300 ft
c) More than 200 ft
d) More than 300 ft

Subject: 10.7.3.6 Use of radar in ATS
Question: 428 / 497 (ID: 1840)

The radar separation minimum may be reduced but not below:

a) 5.0 NM
b) 3.0 NM
c) 1.5 NM
d) 2.0 NM

Subject: 10.7.3.6 Use of radar in ATS
Question: 429 / 497 (ID: 1841)

Unless otherwise prescribed by the appropriate ATS authority, the radar controller should notify the non-radar controller when an aircraft making a radar approach is approximately:

a) 8 NM
b) 10 NM
c) 5 NM
d) 6 NM

Subject: 10.7.3.6 Use of radar in ATS
Question: 430 / 497 (ID: 1842)

An aircraft making a radar approach should be advised to consider executing a missed approach, if the position or identification of the aircraft is in doubt during any portion of the final approach or if the aircraft is not visible on the radar display for significant interval during the last:

a) 2 NM
b) 1 NM
c) 4 NM
d) 3 NM

Subject: 10.7.3.6 Use of radar in ATS
Question: 431 / 497 (ID: 1843)

When conducting a surveillance radar approach, the radar controller shall terminate the surveillance radar approach, except as determined by the appropriate ATS authority, at a distance of:

a) 2 NM from touchdown
b) 3 NM from touchdown
c) 2.5 NM from touchdown
d) 1 NM from touchdown

Subject: 10.7.3.6 Use of radar in ATS
Question: 432 / 497 (ID: 1845)

The radar controller shall not request the pilot to adjust the speed where the aircraft has passed:

a) 5 NM from the threshold on final approach
b) 4 NM from the threshold on final approach
c) 3 NM from the threshold on final approach
d) 2 NM from the threshold on final approach

Subject: 10.7.3.6 Use of radar in ATS
Question: 433 / 497 (ID: 3286)

Radar controlled aircraft on intermediate or final approach may be requested to make minor speed adjustments by ATC. These adjustments shall never be more than:

a) 20 knots and not within 4 NM of threshold
b) 15 knots at any stage
c) 25 knots at any stage
d) 10 knots and not within 5 NM of threshold

Subject: 10.7.3.6 Use of radar in ATS
Question: 434 / 497 (ID: 3297)

Radar identification of a departing aircraft can be achieved if a radar blip is observed within a certain distance from the end of the runway. Identification has to be achieved within:

a) 5NM
b) 3NM
c) 1NM
d) 2NM

Subject: 10.7.3.6 Use of radar in ATS
Question: 435 / 497 (ID: 3309)

When the transponder appears to be unserviceable prior to departure and restorage is impossible, than:

a) the flight can only continue in the most direct manner
b) you must indicate the failure in the fightplan, after which the ATC will endeavour to provide for continuation of the flight
c) departure to the nearest suitable airport where repair can be effected is allowed
d) you are not allowed to commence the flight

Subject: 10.7.3.6 Use of radar in ATS
Question: 436 / 497 (ID: 3320)

Except when prescribed in procedures or made possible by agreements, aircraft under radar-control shall not be vectored closer to the boundary of controlled airspace than:

a) 3 NM
b) 5 NM
c) 1,5 NM
d) 2,5 NM

Subject: 10.7.3.6 Use of radar in ATS
Question: 437 / 497 (ID: 3321)

During radar-control, a "radar-controller" shall issue a missed-approach instruction, in case the "tower-controller" has not issued a "landing-clearance" at the moment the aircraft is:

a) 3 NM from touch-down
b) 4 NM from touch-down
c) 1NM from touch-down
d) 2 NM from touch-down

Subject: 10.7.3.6 Use of radar in ATS
Question: 438 / 497 (ID: 4631)

When a surveillance radar approaches is to be continued to the threshold of the runway transmission should not be interrupted for intervals of more than five seconds while the aircraft is within a distance of:

a) 1.5 NM from the touchdown
b) 2 NM from the touchdown
c) 3 NM from the touchdown
d) 4 NM from the touchdown

Subject: 10.7.3.6 Use of radar in ATS
Question: 439 / 497 (ID: 4632)

The surveillance radar approach shall be terminated at a distance of 2 NM from the touchdown except when as determined by the appropriate ATS authority, the accuracy of the radar equipment permits to be continued to a prescribed point less than 2 NM from the touchdown. In this case distance and level information shall be given at each

a) half mile
b) half NM
c) 1.5 NM
d) 1 NM

Subject: 10.7.3.6 Use of radar in ATS
Question: 440 / 497 (ID: 4633)

Clearence to land or any alternative clearence received from the non-radar controller should normally be passed to the aircraft before it reaches a distance of

a) 4 NM from touchdown
b) 5 NM from touchdown
c) 2 NM from touchdown
d) 3 NM from touchdown

Subject: 10.7.3.6 Use of radar in ATS
Question: 441 / 497 (ID: 4634)

An aircraft making a radar approach should be directed to execute a missed approach if no clearence to land has been received from the non-radar controller by the time the aircraft reaches a distance of:

a) 4 NM from the touchdown
b) 2 NM from the touchdown
c) 1.5 NM from the touchdown
d) 5 NM from the touchdown

Subject: 10.7.3.6 Use of radar in ATS
Question: 442 / 497 (ID: 4635)

An aircraft making a radar approach should be directed to consider executing a missed approach if the aircraft is not visible on the radar display for any significant interval during the:

a) Last 5 NM of the approach
b) Last 2 NM of the approach
c) Last 4 NM of the approach
d) Last 3 NM of the approach

Subject: 10.7.3.6 Use of radar in ATS
Question: 443 / 497 (ID: 4636)

What is the maximum speed adjustment that a pilot should be requested to make when under radar control and established on intermediate and final approach?

a) ± 10KT
b) ± 15 KT
c) ± 25 KT
d) ± 20KT

Subject: 10.7.3.6 Use of radar in ATS
Question: 444 / 497 (ID: 5313)

When a RADAR operator says the following to an aircraft: "fly heading 030", the pilot must fly heading:

a) 030° true, in still air conditions (thereby flying the true track)
b) 030° true
c) 030° magnetic
d) 030° magnetic in still air conditions (thereby flying the magnetic track)

Subject: 10.8.1 Annex 1
Question: 445 / 497 (ID: 820)

In which section of AIP are contained information elements relating to areas and/or routes for which meteorological service is provided?

a) RAc
b) GEN
c) COM
d) MET

Subject: 10.8.1 Annex 15
Question: 446 / 497 (ID: 821)

In which section of AIP are contained information elements relating to refuelling facilities and limitations on refuelling services?

a) FAL
b) Ad
c) SAR
d) GEN

Subject: 10.8.1 Annex 15
Question: 447 / 497 (ID: 822)

In which section of AIP are contained information elements relating to prohibited, restricted and dangerous areas?

a) ENR
b) MAP
c) AGa
d) GEN

Subject: 10.8.1 Annex 15
Question: 448 / 497 (ID: 823)

A notice containing information concerning flight safety, air navigation, technical, administration or legislative matters and originated at the AIS of a state is called:

a) NOTAM
b) Aeronautical Information Publication (AIP)
c) Aeronautical Information Circular (AIC)
d) AIRAc

Subject: 10.8.1 Annex 15
Question: 449 / 497 (ID: 824)

A notice providing information on Rules of the Air, Air Traffic Services and Air Navigation Procedures and distributed in advance of its effective date is:

a) An AIRAc
b) A NOTAM RAc
c) An Advisory NOTAM
d) An ATS NOTAM

Subject: 10.8.1 Annex 15
Question: 450 / 497 (ID: 825)

Each contracting state shall provide an Aeronautical Information Service (AIS) in its territory and for areas in which the state is responsible for the Air Traffic Services outside its territory, and this shall include the preparation and origination of:

a) Only NOTAM's and Circulars
b) AIP, NOTAM's, Circular and AIRAc
c) Only AIP and NOTAM's
d) Integrated Aeronautical Information Package

Subject: 10.8.1 Annex 15
Question: 451 / 497 (ID: 3279)

In which chapter of the AIP can you find a list with "location indicators"?

a) AGA
b) AD
c) GEN
d) ENR

Subject: 10.8.1 Annex 15
Question: 452 / 497 (ID: 3306)

The closure of a runway for a year, because of maintenance, will be published:

a) NOTAM, AIP and MAL
b) in NOTAM and AIP, inclusive Supplement
c) only in NOTAM
d) only in AIP

Subject: 10.8.1 Annex 15
Question: 453 / 497 (ID: 5118)

An integrated aeronautical information package consists of the following elements

a) AIP, including amendment service; supplements to AIP, NOTAM, AIC and checklist summaries
b) AIP including amendment service; supplements to AIP; NOTAM, AIC; AIRAC
c) AIP, including amendment service; supplements to AIP; NOTAM and pre-flight information bulletin (PIB); AIC; checklists and summaries
d) AIP, supplements to AIP; NOTAM and PIB; AIC and checklist summaries

Subject: 10.8.1 Annex 15
Question: 454 / 497 (ID: 5119)

The identification of each prohibited, restricted and danger area shall be composed by:

a) The letters P (Prohibited), R (Restricted) and D (Dangerous) followed by figures
b) The nationality letters for the location indicators assigned to the state, followed by P, R and D
c) The nationality letters for location indicators assigned to the state or territory, followed the letters P. R and D and figures
d) The letters P (Prohibited), R (Restricted) and D (Dangerous) for the area concerned and figures

Subject: 10.8.1 Annex 15
Question: 455 / 497 (ID: 5123)

A checklist of AIP supplements currently in force shall be issued at intervals of:

a) Not more than 28 days
b) Not more than one month
c) Not more than 2 months
d) Not more than three months

Subject: 10.8.1 Annex 15
Question: 456 / 497 (ID: 5299)

An AIRAC is:

a) A notice distributed by means of telecommunication containing information concerning the establishment, condition or change in any aeronautical facility service, procedure or hazard, the timely knowledge of which is essential to personnel concerned with flight operations
b) A package which consists of the following elements: AIP, supplements to the AIP, NOTAM, AIC, checklists and summaries
c) A publication issued by or with the authority of a state containing aeronautical information of a lasting character essential to air navigation
d) An Acronym for a system aimed at advance notification based on common effective dates, of circumstances necessitating significant changes in operating procedures

Subject: 10.8.1 Annex 15
Question: 457 / 497 (ID: 5937)

Select the acronym corresponding to the following definition: an special NOTAM series notifying, by means of a specific format, an important change for the aircraft operations, due to a volcano activity, a volcano eruption or a volcanic ash cloud

a) ASHTAM
b) GVATAM
c) VULTAM
d) NAVTAM

Subject: 10.8.1 Annex 15
Question: 458 / 497 (ID: 5938)

The system notifying in advance the circumstances requiring important changes in the methods of operation, based on common effective dates, is identified by the acronym:

a) EATCHIP
b) AIRAC
c) IFPS
d) NOTAM

Subject: 10.8.1 Annex 15
Question: 459 / 497 (ID: 5939)

The temporary, long-term modification (3 months or more) and the short-term extensive or graphical information are published as follows:

a) Trigger NOTAM
b) AIP Supplements
c) AIP Amendments
d) NOTAM

Subject: 10.9.1.1 Aerodrome data
Question: 460 / 497 (ID: 369)

According to the "Aerodrome Reference Code", the "Code Letter E" shall identify an aircraft wing span of:

a) 52 m up to but not including 65 m
b) 24 m up to but not including 36 m
c) 15 m up to but not including 24 m
d) 36 m up to but not including 52 m

Subject: 10.9.1.1 Aerodrome data
Question: 461 / 497 (ID: 370)

In the "Aerodrome Reference Code" the code element 2 shall identify:

a) The lenght of the aircraft fuselage
b) Only the aircraft wing span
c) The aircraft wing span and the outer main gear wheel span
d) The width of the aircraft wing

Subject: 10.9.1.1 Aerodrome data
Question: 462 / 497 (ID: 371)

The "Aerodrome Reference Code" is a code composed of two elements which are related to the aeroplane performance characteristics and dimensions. These elements are a combination of a number and a letter as in the example under listed:

a) 5E
b) 6d
c) 4F
d) 2b

QDB Information: In principle, answer A) is also correct. However, the letter F means aeroplanes of a wingspan of 65 m and more, and <80 m. Considering the fact, that the B 747-400 has a wingspan of 64.31m, it becomes clear where the 65m limit originates from. This also means probably, that there are presently no airports with code "4F" because there are no aeroplanes requiring this code. Therefore, answer D) is more correct for the time being.

Subject: 10.9.1.1 Aerodrome data
Question: 463 / 497 (ID: 372)

According with the "Aerodrome Reference Code" the "Code number 4" shall identify an aircraft reference field length of:

a) 1 500 m
b) 1 800 m and over
c) 1 200 m
d) 1 600 m

Subject: 10.9.1.1 Aerodrome data
Question: 464 / 497 (ID: 373)

The STOPWAY is a defined rectangular area on the ground at the end of take-off run available prepared as a suitable area where:

a) A landing aircraft can be stopped only in emergency
b) A landing aircraft can be stopped if overcoming the end of runway
c) An aircraft taking-off or landing can be stopped
d) An aircraft can be stopped in the case of an abandoned take-off

Subject: 10.9.1.1 Aerodrome data
Question: 465 / 497 (ID: 374)

"ASDA" (Acceleration Stop Distance Available) is:

a) The length of the take-off run available plus the length of stopway (if stopway provided)
b) The length of the runway plus the length of stopway available (if stopway provided)
c) The length of the take-off run available plus the length of stopway and clearway (if provided)
d) The length of the take-off run available plus the length of the clearway

Subject: 10.9.1.1 Aerodrome data
Question: 466 / 497 (ID: 826)

"Instrument runways" are the following runways intended for the operation of aircraft using instrument approach procedures.

a) Non precision approach runways, precision approach runways category I, II and III
b) Instrument approach runways, precision approach runways category I, II and III
c) Precision approach runways in general
d) Precision approach runways category I, II and III

Subject: 10.9.1.1 Aerodrome data
Question: 467 / 497 (ID: 827)

"Code letter D" shall be chosen to identify a taxiway used by aircraft having an outer main gear wheel span of less than 9 m. The taxiway width shall be:

a) 23 m
b) 15 m
c) 25 m
d) 18 m

Subject: 10.9.1.1 Aerodrome data
Question: 468 / 497 (ID: 828)

"TODA" take-off distance available is:

a) The length of the runway available plus the length of clearway available (if provided)
b) The length of the take-off run available plus the length of clearway available (if provided)
c) The length of the take-off run available plus the length of the stopway
d) The length of the take-off run available plus the length of the stopway and clearway (if provided)

Subject: 10.9.1.1 Aerodrome data
Question: 469 / 497 (ID: 829)

"Clearway" is a defined rectangular area established to:

a) Permit aircraft to make a portion of its initial climb to a specific height
b) Protect aircraft during take-off or landing operations
c) Reduce the risk of damage to aircraft running off a runway
d) Permit the aircraft to stop if it fails the take-off

Subject: 10.9.1.1 Aerodrome data
Question: 470 / 497 (ID: 830)

Which "code letter" shall be chosen to identify a taxiway to be used by an aircraft having a wheel base of 15 m?

a) Code letter "C"
b) Code letter "E"
c) Code letter "D"
d) Code letter "B"

Subject: 10.9.1.2 Visual aids for navigation
Question: 471 / 497 (ID: 831)

How many red lights must a pilot see, whose aircraft, in final approach, is following a normal glide path defined by a PAPI?

a) None
b) 2
c) 3
d) 1

Subject: 10.9.1.2 Visual aids for navigation
Question: 472 / 497 (ID: 832)

Taxiway centre line lights other than an exit taxiway shall be:

a) Fixed lights showing blue
b) Fixed lights showing yellow
c) Fixed lights showing white
d) Fixed lights showing green

Subject: 10.9.1.2 Visual aids for navigation
Question: 473 / 497 (ID: 833)

"3-BAR AVASIS", "PAPI" and "T-VASIS" shall be provided for aircraft having eye-to-wheel heights, when in the flare attitude, not exceeding:

a) Approximately 18 m
b) Approximately 16 m
c) Approximately 15 m
d) Approximately 14 m

Subject: 10.9.1.2 Visual aids for navigation
Question: 474 / 497 (ID: 834)

In a precision approach category I lighting system, the centre line and crossbar lights shall be:

a) Fixed lights showing variable green
b) Flashing lights showing variable white
c) Flashing lights showing variable green
d) Fixed lights showing variable white

Subject: 10.9.1.2 Visual aids for navigation
Question: 475 / 497 (ID: 835)

The abbreviation PAPI stands for:

a) Precision Approach Power Index
b) Precision Approach Path Index
c) Precision Approach Power Indicator
d) Precision Approach Path Indicator

Subject: 10.9.1.2 Visual aids for navigation
Question: 476 / 497 (ID: 836)

The "PAPI" shall consist of:

a) Two wing bars of 4 sharp transition multi-lamp or paired units equally spaced
b) A wing bar of 2 sharp transition multi-lamp equally spaced
c) Two wing bars of 6 sharp transition multi-lamp or paired units equally spaced
d) A wing bar of 4 sharp transition multi-lamp or paired units equally spaced

Subject: 10.9.1.2 Visual aids for navigation
Question: 477 / 497 (ID: 837)

In the "PAPI" system the pilot during an approach will see the two units nearest the runway as red and the two units farthest from the runway as white when:

a) Only on the approach slope
b) Above the approach slope
c) Below the approach slope
d) On or close to the approach slope

Subject: 10.9.1.2 Visual aids for navigation
Question: 478 / 497 (ID: 838)

In the case of parallel runways, each runway designation number shall be supplemented:

a) By a number like "0" and "01" for 2 parallel runways
b) By a letter - for example 2 parallel runways "L" and "R" - for 3 "L", "C" and "R"
c) By a letter for 2 parallel runways
d) By a letter - for example 3 parallel runways "L" and "R" and the central has no letter

Subject: 10.9.1.2 Visual aids for navigation
Question: 479 / 497 (ID: 839)

Taxiway edge lights shall be:

a) Flashing showing blue
b) Fixed showing green
c) Fixed showing blue
d) Fixed showing yellow

Subject: 10.9.1.2 Visual aids for navigation
Question: 480 / 497 (ID: 840)

Runway end lights shall be:

a) Fixed lights showing variable white
b) Fixed unidirectional lights showing red in the direction of the runway
c) Fixed lights showing variable red
d) Fixed unidirectional lights showing white in the direction of the runway

Subject: 10.9.1.2 Visual aids for navigation
Question: 481 / 497 (ID: 841)

Runway threshold lights shall be:

a) Fixed lights showing green or white colours
b) Fixed lights green colours
c) Fixed unidirectional lights showing green in the direction of approach to the runway
d) Fixed unidirectional lights showing white in the direction of approach to the runway

Subject: 10.9.1.2 Visual aids for navigation
Question: 482 / 497 (ID: 842)

Runway edge lights excepted in the case of a displaced threshold shall be:

a) Fixed lights, white or yellow colour
b) Fixed lights showing variable white
c) Fixed lights showing variable white or yellow
d) Flashing white

Subject: 10.9.1.2 Visual aids for navigation
Question: 483 / 497 (ID: 843)

Runway threshold identification lights, when provided, should be:

a) Flashing green
b) Fixed green
c) Fixed white
d) Flashing white

Subject: 10.9.1.2 Visual aids for navigation
Question: 484 / 497 (ID: 844)

The light shown by an "Aerodrome Identification Beacon" at a land aerodrome shall be:

a) Green colour identification given by Morse Code
b) White colour identification given by Morse Code
c) White and green colour identification given by Morse Code
d) Blue colour identification given by Morse Code

Subject: 10.9.1.2 Visual aids for navigation
Question: 485 / 497 (ID: 845)

In the "VASIS" , how many light units are in each wing bar?

a) 3
b) 4
c) 5
d) 2

Subject: 10.9.1.2 Visual aids for navigation
Question: 486 / 497 (ID: 846)

In a precision approach category I, lighting system, the single, two and three light sources on the centre line have a length of:

a) 150 m
b) 300 m
c) 200 m
d) 250 m

Subject: 10.9.1.2 Visual aids for navigation
Question: 487 / 497 (ID: 3287)

What is the length of an approach lighting system of a precision-approach runway CAT II:

a) 150m
b) 600m
c) 300m
d) 900m

Subject: 10.9.1.2 Visual aids for navigation
Question: 488 / 497 (ID: 3288)

What is a "barrette"?

a) a frangible structure on which approach lights are fixed
b) three or more groundlights closely spaced together to appear as a bar of lights
c) a highted obstacle near the runway and/or taxiway
d) a CAT II or III holding position

Subject: 10.9.1.2 Visual aids for navigation
Question: 489 / 497 (ID: 3289)

A precision approach runway CAT. II is an instrument runway served by ILS and visual aids intended for operations down to:

a) a RVR of 550 meters and a DH of not less than 200 ft
b) a RVR of 200 meters and a DH of not less than 100 ft
c) a RVR of 300-450 meters and a DH of not less than 100 ft
d) a RVR of 250 meters and a DH of not less than 200 ft

Subject: 10.9.1.2 Visual aids for navigation
Question: 490 / 497 (ID: 3291)

When a fixed-distance marking has to be provided this marking shall commence at

a) 450 m from threshold
b) 300 m from threshold
c) 150 m from threshold
d) 600 m from threshold

Subject: 10.9.1.2 Visual aids for navigation
Question: 491 / 497 (ID: 3298)

Runway-lead-in lighting should consist:

a) of flashing lights only
b) of group of at least three white lights flashing in sequence towards the runway
c) always of a straight row of lights towards the runway
d) of an arbitrary amount of green lights

Subject: 10.9.1.2 Visual aids for navigation
Question: 492 / 497 (ID: 3307)

Aerodromes signs should be in the following configuration:

a) information signs; yellow or black background with black or yellow inscriptions
b) information signs; orange background with black inscriptions
c) mandatory instruction signs; red background with black inscriptions
d) mandatory instruction signs; black background with red inscriptions

Subject: 10.9.1.3 Visual aids for denoting obstacles
Question: 493 / 497 (ID: 847)

High intensity obstacle lights should be:

a) Flashing white
b) Fixed orange
c) Fixed red
d) Flashing red

Subject: 10.9.1.3 Visual aids for denoting obstacles
Question: 494 / 497 (ID: 848)

Low intensity obstacle lights on mobile objects shall be:

a) Flashing blue
b) Fixed red or preferably orange
c) Fixed red or preferably blue
d) Flashing red or preferably yellow

Subject: 10.9.1.3 Visual aids for denoting obstacles
Question: 495 / 497 (ID: 849)

Low intensity obstacle lights on fixed objects shall be:

a) Fixed orange
b) Flashing yellow
c) Flashing red
d) Fixed red

Subject: 10.9.1.4 Visual aids for denoting restricted use of areas
Question: 496 / 497 (ID: 850)

The runway edge lights shall be:

a) green
b) white
c) blue
d) red

Subject: 10.9.1.5 Emergency and other services
Question: 497 / 497 (ID: 852)

The aerodrome category for rescue and fire fighting is based on:

a) The over-all length of the longest aeroplane normally using the aerodrome and its maximum fuselage width
b) The over-all length of the longest aeroplane normally using the aerodrome and its maximum fuselage weight
c) The over-all length of the longest aeroplane
d) The longest aeroplane maximum width only